  - Design and Analysis of a Butterworth IIR Low-Pass Filter Using MATLAB.

clc;

clear;

close all;

%% Filter Specifications

Fs = 1000;        % Sampling Frequency (Hz)

Fp = 100;         % Passband Frequency (Hz)

Fst = 150;        % Stopband Frequency (Hz)

Rp = 1;           % Passband Ripple (dB)

Rs = 40;          % Stopband Attenuation (dB)

%% Normalize Frequencies

Wp = Fp/(Fs/2);    % Normalized Passband Frequency

Ws = Fst/(Fs/2);   % Normalized Stopband Frequency

%% Calculate Filter Order

[n,Wn] = buttord(Wp,Ws,Rp,Rs);

%% Design Butterworth Low-Pass Filter

[b,a] = butter(n,Wn,'low');

%% Display Filter Details

disp('Butterworth Low-Pass Filter');

disp(['Filter Order = ',num2str(n)]);

disp('Numerator Coefficients (b):');

disp(b);

disp('Denominator Coefficients (a):');

disp(a);

%% Frequency Response

figure;

freqz(b,a,1024,Fs);

title('Frequency Response of Butterworth Low-Pass Filter');

%% Magnitude and Phase Response

[H,f] = freqz(b,a,1024,Fs);

figure;

subplot(2,1,1);

plot(f,20*log10(abs(H)),'b','LineWidth',2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Magnitude Response');

subplot(2,1,2);

plot(f,unwrap(angle(H))*180/pi,'r','LineWidth',2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('Phase Response');

%% Impulse Response

figure;

impz(b,a);

grid on;

title('Impulse Response');

%% Pole-Zero Plot

figure;

zplane(b,a);

grid on;

title('Pole-Zero Plot');

%% Filter Coefficients

fprintf('\nNumerator Coefficients (b):\n');

fprintf('%f ',b);

fprintf('\n\nDenominator Coefficients (a):\n');

fprintf('%f ',a);

%% Test Signal

t = 0:1/Fs:1;

% Low Frequency Signal (Passband)

x1 = sin(2*pi*50*t);

% High Frequency Noise (Stopband)

x2 = 0.5*sin(2*pi*250*t);

% Combined Signal

x = x1 + x2;

% Filtered Output

y = filter(b,a,x);

%% Input vs Output

figure;

subplot(2,1,1);

plot(t,x);

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Input Signal');

subplot(2,1,2);

plot(t,y,'r');

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Filtered Output');

%% FFT Analysis

N = length(x);

faxis = (0:N-1)*(Fs/N);

X = abs(fft(x));

Y = abs(fft(y));

figure;

subplot(2,1,1);

plot(faxis,X);

xlim([0 500]);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('Spectrum of Input Signal');

subplot(2,1,2);

plot(faxis,Y,'r');

xlim([0 500]);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('Spectrum of Filtered Signal');

OUTPUTS: