4.2. Design and Analysis of an FIR Low-Pass       Filter Using the Hamming  Window Technique in      MATLAB.

Programming Code:

clc;

clear;

close all;

%% Filter Specifications

Fs = 1000;              % Sampling Frequency (Hz)

Fc = 100;               % Cutoff Frequency (Hz)

N = 50;                 % Filter Order

%% Normalized Cutoff Frequency

Wn = Fc/(Fs/2);

%% Design FIR Low-Pass Filter using Hamming Window

b = fir1(N,Wn,'low',hamming(N+1));

a = 1;

%% Display Filter Details

disp('FIR Low-Pass Filter Using Hamming Window');

disp(['Filter Order = ', num2str(N)]);

disp('Filter Coefficients:');

disp(b);

%% Frequency Response

figure;

freqz(b,a,1024,Fs);

title('Frequency Response of FIR Low-Pass Filter (Hamming Window)');

%% Magnitude and Phase Response

[H,f] = freqz(b,a,1024,Fs)

figure;

subplot(2,1,1);

plot(f,20*log10(abs(H)),'b','LineWidth',2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Magnitude Response');

subplot(2,1,2);

plot(f,unwrap(angle(H))*180/pi,'r','LineWidth',2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('Phase Response');

%% Impulse Response

figure;

impz(b,a);

grid on;

title('Impulse Response');

%% Filter Coefficients

figure;

stem(0:N,b,'filled');

grid on;

xlabel('Sample Number');

ylabel('Amplitude');

title('FIR Filter Coefficients');

%% Pole-Zero Plot

figure;

zplane(b,a);

grid on;

title('Pole-Zero Plot');

%% Test Signal

t = 0:1/Fs:1;

% Desired Signal (50 Hz)

x1 = sin(2*pi*50*t);

% Noise Signal (250 Hz)

x2 = 0.5*sin(2*pi*250*t);

% Combined Signal

x = x1 + x2;

%% Apply Filter

y = filter(b,a,x);

%% Input and Output Signals

figure;

subplot(2,1,1);

plot(t,x,'b');

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Input Signal');

subplot(2,1,2);

plot(t,y,'r');

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Filtered Output Signal');

%% FFT Analysis

L = length(x);

X = abs(fft(x));

Y = abs(fft(y));

faxis = (0:L-1)*(Fs/L);

figure;

subplot(2,1,1);

plot(faxis,X,'b');

xlim([0 500]);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('Input Signal Spectrum');

subplot(2,1,2);

plot(faxis,Y,'r');

xlim([0 500]);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('Filtered Signal Spectrum');

%% Display Filter Information

fprintf('\nSampling Frequency : %.0f Hz\n',Fs);

fprintf('Cutoff Frequency   : %.0f Hz\n',Fc);

fprintf('Filter Order       : %d\n',N);

fprintf('Window Used        : Hamming Window\n');

OUTPUT: