3.1 Design and Comparative Analysis of Butterworth and Chebyshev Type-I IIR Band-Pass Filters Using MATLAB

Programming Code:

clc;

clear;

close all;

%% Filter Specifications

Fs = 1000;          % Sampling Frequency (Hz)

Fp1 = 100;          % Lower Passband Frequency (Hz)

Fp2 = 300;          % Upper Passband Frequency (Hz)

Fs1 = 80;           % Lower Stopband Frequency (Hz)

Fs2 = 350;          % Upper Stopband Frequency (Hz)

Rp = 1;             % Passband Ripple (dB)

Rs = 40;            % Stopband Attenuation (dB)

%% Normalize Frequencies

Wp = [Fp1 Fp2]/(Fs/2);

Ws = [Fs1 Fs2]/(Fs/2);

%% Butterworth Band-Pass Filter

[Nb,Wnb] = buttord(Wp,Ws,Rp,Rs);

[bb,ab] = butter(Nb,Wnb,'bandpass');

%% Chebyshev Type-I Band-Pass Filter

[Nc,Wnc] = cheb1ord(Wp,Ws,Rp,Rs);

[bc,ac] = cheby1(Nc,Rp,Wnc,'bandpass');

%% Display Filter Orders

disp('--------------------------------');

disp('Butterworth Filter');

disp(['Order = ',num2str(Nb)]);

disp('--------------------------------');

disp('Chebyshev Type-I Filter');

disp(['Order = ',num2str(Nc)]);

%% Frequency Response Comparison

figure;

freqz(bb,ab,1024,Fs);

title('Butterworth Band-Pass Filter');

figure;

freqz(bc,ac,1024,Fs);

title('Chebyshev Type-I Band-Pass Filter');

%% Magnitude Comparison

[Hb,f] = freqz(bb,ab,1024,Fs);

[Hc,~] = freqz(bc,ac,1024,Fs);

figure;

plot(f,20*log10(abs(Hb)),'b','LineWidth',2);

hold on;

plot(f,20*log10(abs(Hc)),'r--','LineWidth',2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Magnitude Response Comparison');

legend('Butterworth','Chebyshev-I');

%% Phase Comparison

figure;

plot(f,unwrap(angle(Hb))*180/pi,'b','LineWidth',2);

hold on;

plot(f,unwrap(angle(Hc))*180/pi,'r--','LineWidth',2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('Phase Response Comparison');

legend('Butterworth','Chebyshev-I');

%% Pole-Zero Plots

figure;

subplot(1,2,1);

zplane(bb,ab);

title('Butterworth Pole-Zero Plot');

grid on;

subplot(1,2,2);

zplane(bc,ac);

title('Chebyshev-I Pole-Zero Plot');

grid on;

%% Impulse Responses

figure;

subplot(2,1,1);

impz(bb,ab);

title('Butterworth Impulse Response');

grid on;

subplot(2,1,2);

impz(bc,ac);

title('Chebyshev-I Impulse Response');

grid on;

%% Test Signal

t = 0:1/Fs:1;

x = sin(2*pi*50*t) + ...

sin(2*pi*200*t) + ...

sin(2*pi*400*t);

%% Filter Output

yb = filter(bb,ab,x);

yc = filter(bc,ac,x);

%% Time Domain Comparison

figure;

subplot(3,1,1);

plot(t,x);

grid on;

title('Input Signal');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(3,1,2);

plot(t,yb,'b');

grid on;

title('Butterworth Output');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(3,1,3);

plot(t,yc,'r');

grid on;

title('Chebyshev-I Output');

xlabel('Time (s)');

ylabel('Amplitude');

%% FFT Comparison

N = length(x);

X = abs(fft(x));

Yb = abs(fft(yb));

Yc = abs(fft(yc));

faxis = (0:N-1)*(Fs/N);

figure;

subplot(3,1,1);

plot(faxis,X);

xlim([0 500]);

grid on;

title('Input Spectrum');

subplot(3,1,2);

plot(faxis,Yb,'b');

xlim([0 500]);

grid on;

title('Butterworth Output Spectrum');

subplot(3,1,3);

plot(faxis,Yc,'r');

xlim([0 500]);

grid on;

title('Chebyshev-I Output Spectrum');

OUTPUTS: