2.Chebyshev IIR Filters 2.1 Design and performance Evaluation of a Chebyshev Type I IIR Low Pass Filter using MATLAB

Programming Code:

clc;

clear;

close all;

%% Filter Specifications

Fs = 1000;      % Sampling Frequency (Hz)

Fp = 100;       % Passband Frequency (Hz)

Fst = 150;      % Stopband Frequency (Hz)

Rp = 1;         % Passband Ripple (dB)

Rs = 40;        % Stopband Attenuation (dB)

%% Normalize Frequencies

Wp = Fp/(Fs/2);

Ws = Fst/(Fs/2);

%% Calculate Minimum Filter Order

[n,Wn] = cheb1ord(Wp,Ws,Rp,Rs);

%% Design Chebyshev Type-I Low Pass Filter

[b,a] = cheby1(n,Rp,Wn,'low');

%% Display Filter Information

disp('Chebyshev Type-I IIR Low Pass Filter');

disp(['Filter Order = ',num2str(n)]);

disp('Numerator Coefficients (b):');

disp(b);

disp('Denominator Coefficients (a):');

disp(a);

%% Frequency Response

figure;

freqz(b,a,1024,Fs);

title('Frequency Response of Chebyshev Type-I Low Pass Filter');

%% Magnitude and Phase Response

[H,f] = freqz(b,a,1024,Fs);

figure;

subplot(2,1,1);

plot(f,20*log10(abs(H)),'b','LineWidth',2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Magnitude Response');

subplot(2,1,2);

plot(f,unwrap(angle(H))*180/pi,'r','LineWidth',2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('Phase Response');

%% Impulse Response

figure;

impz(b,a);

grid on;

title('Impulse Response');

%% Pole-Zero Plot

figure;

zplane(b,a);

grid on;

title('Pole-Zero Plot');

%% Test Signal

t = 0:1/Fs:1;

% Passband Signal

x1 = sin(2*pi*50*t);

% Stopband Signal

x2 = 0.5*sin(2*pi*250*t);

% Combined Signal

x = x1 + x2;

%% Filter Output

y = filter(b,a,x);

%% Time Domain Signals

figure;

subplot(2,1,1);

plot(t,x,'b');

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Input Signal');

subplot(2,1,2);

plot(t,y,'r');

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Filtered Output Signal');

%% FFT Analysis

N = length(x);

X = abs(fft(x));

Y = abs(fft(y));

faxis = (0:N-1)*(Fs/N);

figure;

subplot(2,1,1);

plot(faxis,X,'b');

xlim([0 500]);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('Spectrum of Input Signal');

subplot(2,1,2);

plot(faxis,Y,'r');

xlim([0 500]);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('Spectrum of Filtered Signal');

%% Display Filter Coefficients

fprintf('\nNumerator Coefficients (b):\n');

disp(b);

fprintf('Denominator Coefficients (a):\n');

disp(a);

OUTPUTS: