4.1. Design and Analysis of an FIR Low-Pass Filter Using the Rectangular Window Technique in MATLAB.

clc;

clear;

close all;

%% Filter Specifications

Fs = 1000;          % Sampling Frequency (Hz)

Fc = 100;           % Cutoff Frequency (Hz)

N = 50;             % Filter Order

%% Normalized Cutoff Frequency

Wn = Fc/(Fs/2);

%% Design FIR Low-Pass Filter using Rectangular Window

b = fir1(N,Wn,'low',rectwin(N+1));

a = 1;

disp('FIR Low-Pass Filter using Rectangular Window');

disp('Filter Coefficients:');

disp(b);

%% Frequency Response

figure;

freqz(b,a,1024,Fs);

title('Frequency Response of FIR Low-Pass Filter');

%% Magnitude and Phase Response

[H,f] = freqz(b,a,1024,Fs);

figure;

subplot(2,1,1);

plot(f,20*log10(abs(H)),'b','LineWidth',2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Magnitude Response');

subplot(2,1,2);

plot(f,unwrap(angle(H))*180/pi,'r','LineWidth',2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('Phase Response');

%% Impulse Response

figure;

impz(b,a);

grid on;

title('Impulse Response');

%% Filter Coefficients (Stem Plot)

figure;

stem(0:N,b,'filled');

grid on;

xlabel('Sample Number');

ylabel('Amplitude');

title('Filter Coefficients');

%% Pole-Zero Plot

figure;

zplane(b,a);

grid on;

title('Pole-Zero Plot');

%% Generate Test Signal

t = 0:1/Fs:1;

% Low-frequency signal

x1 = sin(2*pi*50*t);

% High-frequency noise

x2 = 0.5*sin(2*pi*250*t);

% Combined signal

x = x1 + x2;

%% Apply FIR Filter

y = filter(b,a,x);

%% Time Domain Signals

figure;

subplot(2,1,1);

plot(t,x,'b');

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Input Signal');

subplot(2,1,2);

plot(t,y,'r');

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Filtered Output Signal');

%% FFT Analysis

Nfft = length(x);

X = abs(fft(x));

Y = abs(fft(y));

faxis = (0:Nfft-1)*(Fs/Nfft);

figure;

subplot(2,1,1);

plot(faxis,X,'b');

grid on;

xlim([0 500]);

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('Spectrum of Input Signal');

subplot(2,1,2);

plot(faxis,Y,'r');

grid on;

xlim([0 500]);

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('Spectrum of Filtered Signal');

%% Display Filter Information

fprintf('\nFilter Order = %d\n',N);

fprintf('Cutoff Frequency = %.2f Hz\n',Fc);

fprintf('Sampling Frequency = %.2f Hz\n',Fs);

OUTPUT: